# Case Study 1 — Video Conferencing Quality Degradation

## Case Problem

A cloud-based video conferencing platform used by a multinational company shows voice distortion, frozen video, and delayed communication during peak hours. Monitoring reports show 8% packet loss, 180 ms jitter, and fluctuating bandwidth utilization. The administrator suspects the transport protocol or the application-layer buffering strategy.

## Q1. Is the problem caused by the transport protocol (TCP/UDP) or the application layer?

Analysis

Real-time video conferencing is normally carried over UDP (via RTP/RTCP), not TCP, because live audio/video cannot tolerate the retransmission delay and head-of-line blocking that TCP's reliability mechanism introduces. The reported symptoms must therefore be examined against what UDP itself can and cannot control:

- 8% packet loss: UDP does not retransmit lost packets, so this loss is expected to surface directly as glitches unless the application layer conceals it.

- 180 ms jitter: this is a network-layer variation in inter-packet arrival time; UDP has no jitter buffer of its own — jitter compensation is strictly an application-layer responsibility.

- Frozen frames and delayed audio: this pattern (freeze-then-catch-up) is the signature of an under-sized or poorly-adaptive de-jitter buffer, not of TCP retransmission stalls, because the case explicitly describes a UDP-style real-time service.

Conclusion of analysis: the transport protocol (UDP) is behaving exactly as designed — it is delivering packets on a best-effort basis without guaranteeing order, delivery, or timing. The actual degradation the users experience is dominated by how the application layer (the conferencing client) buffers, conceals loss, and adapts its codec to these UDP-layer conditions. The root cause is therefore primarily an application-layer buffering/adaptation problem, triggered by underlying UDP characteristics that the application has not been tuned to absorb.

## Q2. Justification based on real-time multimedia communication requirements

Justification

Real-time multimedia has two requirements that conflict directly with TCP's design:

| Requirement | TCP behaviour | UDP behaviour | Why it matters here |
| --- | --- | --- | --- |
| Timeliness | Retransmits lost segments, increasing delay | No retransmission — data is either on time or dropped | 180 ms jitter shows the network is already timing-sensitive; TCP retransmission would add further delay |
| Ordered delivery | Strictly in-order; one lost segment blocks all following ones (head-of-line blocking) | Delivers packets as they arrive, out of order if needed | A single lost audio packet must not stall the whole stream |
| Loss tolerance | Zero loss tolerated — must be recovered | Loss tolerated; concealed by codec/jitter buffer | 8% loss is survivable for audio/video if concealed properly, but fatal for TCP throughput |
| Congestion response | Backs off aggressively on loss (halves cwnd) | No built-in backoff (must be added at application layer, e.g., via RTP/RTCP or congestion-controlled variants) | Explains fluctuating bandwidth utilization under peak load |

Because retransmission-based reliability actively harms real-time interactivity (a 200 ms-old retransmitted video frame is useless once the moment has passed), UDP is the technically correct transport choice. The observed 8% loss and 180 ms jitter are consistent with normal best-effort IP transport under peak congestion — they are not evidence of a UDP malfunction, but they do demand strong application-layer compensation, which appears to be missing or under-tuned in this deployment.

## Q3. Recommended protocol-level and application-level improvements

Recommended Solution

- Adaptive jitter buffer: replace any fixed-size buffer with a dynamic de-jitter buffer that resizes based on measured jitter (target ≈ 2–3× the observed 180 ms variance) so late packets are absorbed without excessive added delay.

- Forward Error Correction (FEC): add redundant parity packets at the RTP layer so that a portion of the 8% lost packets can be reconstructed without retransmission.

- Packet-loss concealment (PLC): use codec-level concealment (e.g., waveform substitution for audio, frame extrapolation for video) to mask residual loss after FEC.

- Congestion-controlled RTP (e.g., Google Congestion Control / WebRTC's GCC): dynamically adjust the send bitrate in response to detected loss and jitter, addressing the fluctuating bandwidth utilization.

- DSCP/QoS marking (Expedited Forwarding) on the network so conferencing traffic is prioritized over bulk data traffic during peak hours.

- Scalable codec use (e.g., Opus for audio, VP9/AV1 SVC for video) to gracefully degrade quality rather than freeze when bandwidth drops.

Expected Improvement

With an adaptive jitter buffer and FEC tuned to the measured 180 ms jitter and 8% loss, perceptual audio/video continuity is restored because loss is either recovered or concealed before it reaches the user, and bitrate adaptation prevents the buffer from starving under fluctuating bandwidth.

## Conclusion — Case Study 1

The transport protocol (UDP) is functioning as intended for real-time media; the actual fault lies in insufficient application-layer buffering and adaptation for the measured 8% loss and 180 ms jitter. Combining an adaptive jitter buffer, FEC, packet-loss concealment, and congestion-controlled bitrate adaptation directly targets the reported symptoms and restores conferencing quality.

# Case Study 2 — High DNS Resolution Latency (Global Web Services)

## Case Problem

A multinational company's web services suffer slow homepage loads because average DNS lookup time is about 800 ms. The company wants to redesign its DNS infrastructure to cut lookup time below 50 ms while maintaining availability during server failures.

## Q1. Reasons behind the high DNS resolution time

Analysis

- Single, geographically distant authoritative server: if all queries travel to one region, users far from that server incur large round-trip propagation delay — a major contributor to an 800 ms average.

- Deep recursive resolution chain: an uncached query may need to traverse root → TLD → authoritative name server, and each hop adds its own RTT, especially across continents.

- Low or absent caching (TTL issues): if TTL values are too low, resolvers re-query the authoritative server far more often than necessary, repeatedly paying the full resolution cost instead of serving from cache.

- No pre-fetching: without pre-fetching, resolution only starts when the user actually requests the page, so the entire chain's latency lands directly in the critical path of page load.

- No load distribution / Anycast: a single unicast IP address concentrates all global query load onto one physical location, causing queueing delay during peak traffic on top of propagation delay.

## Q2. Proposed DNS architecture (Anycast, pre-fetching, TTL optimization) to reach <50 ms

Proposed Solution

Text-based architecture:

┌───────────────────────────┐

│   Anycast IP (single IP)   │

│  announced from many PoPs  │

└─────────────┬─────────────┘

┌───────────────┬──────────┴───────┬───────────────┐

┌─────▼─────┐   ┌─────▼─────┐      ┌─────▼─────┐   ┌─────▼─────┐

│ DNS PoP   │   │ DNS PoP   │      │ DNS PoP   │   │ DNS PoP   │

│ Americas  │   │  Europe   │      │   Asia    │   │  Oceania  │

│(secondary │   │(secondary │      │(secondary │   │(secondary │

│ auth NS)  │   │ auth NS)  │      │ auth NS)  │   │ auth NS)  │

└─────┬─────┘   └─────┬─────┘      └─────┬─────┘   └─────┬─────┘

└───────────────┴─────────┬─────────┴───────────────┘

│ zone sync (AXFR/IXFR)

┌────────▼────────┐

│ Primary/Hidden   │

│ Authoritative NS │

└──────────────────┘

- Anycast DNS: the same IP address is announced from multiple points of presence (PoPs) worldwide; BGP routes each user's query to the topologically nearest PoP, cutting propagation delay from a global hop to a regional one.

- DNS pre-fetching: the browser/OS resolves likely-needed domains (e.g., linked resources, next-navigation domains) in the background before the user clicks, so the 800 ms cost is hidden outside the critical path for repeat/predicted lookups.

- TTL optimization: set TTL long enough (e.g., 300–3600 s for stable records) that recursive resolvers cache aggressively and avoid re-querying the authoritative layer on every request, while keeping it short enough (not below ~60 s) to allow failover changes to propagate.

- Multiple synchronized authoritative servers per region (hidden primary + secondaries) so no single PoP failure removes a region's DNS capability.

Combined effect: Anycast reduces per-query RTT to a nearby PoP (often 5–20 ms), pre-fetching removes resolution from the user-visible path for anticipated lookups, and TTL tuning maximizes local resolver cache hits — together bringing effective lookup time well below the 50 ms target for the vast majority of queries.

## Q3. Advantages and limitations of the proposed architecture

| Aspect | Advantage | Limitation |
| --- | --- | --- |
| Performance | Queries answered by the nearest PoP; typical latency drops from ~800 ms to tens of ms | Anycast routing depends on BGP convergence, which is outside the DNS operator's direct control |
| Scalability | New PoPs can be added without changing the published IP address | Requires coordination with multiple ISPs/IXPs and ongoing BGP route monitoring |
| Availability | Automatic failover — BGP withdraws the route of a failed PoP, traffic shifts to the next nearest one | Misconfigured route withdrawal can cause a brief black-holing of queries for that region |
| Caching (TTL) | Higher cache-hit ratio reduces authoritative-server load and query latency | Longer TTLs slow propagation of legitimate record changes (e.g., during failover) |
| Pre-fetching | Hides resolution latency from the user for predictable navigation | Wastes some resolver capacity on speculative lookups that are never used |

## Conclusion — Case Study 2

The 800 ms lookup time results from centralized, single-location authoritative resolution with weak caching. Deploying Anycast DNS across multiple PoPs, enabling predictive pre-fetching, and tuning TTLs to balance cache efficiency against failover agility together bring lookup latency below the 50 ms target while preserving high availability.

# Case Study 3 — Distributed DNS for Global Cloud Traffic

## Case Problem

A global cloud provider routes all DNS queries to a single primary DNS server, causing ~800 ms delays during promotional traffic spikes. A highly available, globally distributed DNS solution is required.

## Q1. Impact of centralized DNS architecture on application performance

Analysis

- Single point of failure: if the one primary server becomes unreachable or overloaded, all DNS resolution worldwide fails simultaneously, taking down access to every dependent application.

- Queueing delay under load: during promotional events, the surge in simultaneous queries exceeds the single server's processing capacity, so requests queue and average latency rises sharply — directly explaining the reported spikes.

- Propagation delay for distant users: users far from the server's physical location pay a large fixed RTT cost on every uncached lookup, on top of any queueing delay.

- No geographic load spreading: all global demand converges on one location instead of being distributed, so the server cannot scale horizontally to absorb bursty promotional traffic.

## Q2. Distributed DNS design to minimize latency and maximize availability

Proposed Solution

Users worldwide

│  DNS query to Anycast IP

▼

┌─────────────────────────────────────────────────┐

│        Global Anycast Network (many PoPs)        │

│  ┌───────┐  ┌───────┐  ┌───────┐  ┌───────┐      │

│  │PoP-US │  │PoP-EU │  │PoP-IN │  │PoP-SG │ ...  │

│  │(cache │  │(cache │  │(cache │  │(cache │      │

│  │+ auth)│  │+ auth)│  │+ auth)│  │+ auth)│      │

│  └───┬───┘  └───┬───┘  └───┬───┘  └───┬───┘      │

└──────┼──────────┼──────────┼──────────┼──────────┘

└──────────┴────┬─────┴──────────┘

│ zone transfer / sync

┌────────▼────────┐

│  Hidden Primary   │  (source of truth,

│  Authoritative NS │   not directly queried)

└───────────────────┘

- Multiple geographically distributed authoritative servers, all sharing one Anycast IP, so BGP steers each query to the nearest healthy PoP automatically.

- Hidden-primary/secondary model: a non-public primary holds the source-of-truth zone data and pushes updates to all public-facing secondaries, keeping every PoP consistent without exposing a single attackable/overloadable node.

- Health-checked failover: each PoP is continuously monitored; an unhealthy PoP's BGP route is withdrawn so traffic is absorbed by the remaining PoPs within seconds.

- Aggressive edge caching at recursive resolvers, supported by well-tuned TTLs, to absorb promotional-event traffic spikes without hitting authoritative servers at all for repeat queries.

## Q3. How TTL, Anycast routing, and DNS caching influence performance and fault tolerance

| Mechanism | Effect on Performance | Effect on Fault Tolerance |
| --- | --- | --- |
| TTL values | Longer TTL → higher resolver cache-hit rate → fewer round trips to authoritative servers → lower average latency | Longer TTL → stale data may persist briefly after a failover; must be balanced (e.g., 60–300 s) so recovery propagates quickly |
| Anycast routing | Routes each query to the topologically nearest PoP → latency approaches regional RTT instead of global RTT | If one PoP fails, BGP automatically reroutes queries to the next nearest PoP with no manual intervention |
| DNS caching | Most repeat queries are answered locally at the resolver, never reaching the authoritative layer, keeping the 800 ms-style bottleneck from recurring | Reduces load concentration on authoritative servers during surges, indirectly protecting availability under promotional traffic spikes |

## Conclusion — Case Study 3

Centralizing DNS on a single server creates both a latency bottleneck and a single point of failure, exactly as observed during promotional-event surges. A distributed Anycast architecture with a hidden primary, health-checked secondaries, and well-tuned TTL-driven caching removes the bottleneck, spreads load geographically, and gives automatic, near-instant failover.

# Case Study 4 — TCP Latency in a Stock Trading Platform

## Case Problem

An electronic trading platform's order acknowledgment latency rises from 1 ms to 40 ms during market opening, with increased TCP retransmissions, longer connection establishment times, and excessive buffering.

## Q1. Three TCP-related factors responsible for the latency increase

Analysis

- Flow control (receive window exhaustion): during the opening surge, the receiver's application may not drain its socket buffer fast enough; the TCP receive window shrinks, forcing the sender to pause or send smaller segments, adding delay before each acknowledgment.

- Nagle's algorithm: Nagle's algorithm delays sending small segments until either a full MSS is ready or a pending ACK arrives, so it can hold back the small, latency-critical order messages typical of trading systems, directly adding tens of milliseconds of queuing delay.

- SYN queue limitations (listen backlog): a burst of new connection attempts at market open can exceed the server's SYN backlog queue size; excess SYNs are dropped and must be retransmitted by the client after a timeout, lengthening connection establishment time and contributing to the observed retransmissions.

## Q2. How each factor affects transaction processing in high-frequency systems

| Factor | Mechanism of Impact | Effect on High-Frequency Trading |
| --- | --- | --- |
| Flow control | Shrunken receive window forces sender to throttle or stall segment transmission | Order/acknowledgment messages queue at the sender, directly adding to the 1→40 ms latency jump |
| Nagle's algorithm | Buffers small outgoing segments waiting for more data or an ACK (interacts badly with delayed ACK) | Small, frequent order messages get held for up to one ACK-delay interval, adding non-trivial latency per message at high message rates |
| SYN queue limits | Backlog overflow causes dropped SYNs and forces retransmission with exponential backoff | New client connections at market open take multiples of the base RTT to establish, delaying the first trades of the session |

## Q3. Recommended TCP configuration changes

Recommended Solution

- Disable Nagle's algorithm on trading sockets (TCP_NODELAY) so small order/acknowledgment messages are sent immediately instead of being batched, removing the Nagle–delayed-ACK interaction that adds latency.

- Increase the TCP receive/send buffer sizes and tune the application to drain its socket buffer faster, preventing receive-window shrinkage under burst load.

- Increase the SYN backlog queue (e.g., Linux net.core.somaxconn and net.ipv4.tcp_max_syn_backlog) and enable SYN cookies so opening-bell connection bursts are accepted instead of dropped.

- Enable TCP window scaling to allow larger effective windows for high-bandwidth, low-latency links, reducing flow-control throttling.

- Use connection pooling / persistent connections so trading sessions avoid repeated SYN handshakes during the high-connection-rate opening period.

- Enable selective acknowledgment (SACK) so a single lost segment does not require retransmitting the entire window, reducing retransmission-driven delay.

Expected Improvement

Disabling Nagle's algorithm removes small-message batching delay, a larger SYN backlog absorbs the opening-bell connection surge without drops, and adequate buffer sizing with window scaling prevents flow-control stalls — together these changes are expected to pull acknowledgment latency back down from the observed 40 ms toward the baseline 1 ms.

## Conclusion — Case Study 4

The latency spike at market open is explained by three compounding TCP effects: flow-control throttling from a shrinking receive window, Nagle's-algorithm delay on small order messages, and SYN backlog overflow during the connection surge. Disabling Nagle's algorithm, enlarging buffers and the SYN backlog, and enabling window scaling/SACK directly target each cause and restore low-latency order acknowledgment.