SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools.

(BL4,BL5)

Assessment Tool Weightage: 30%

Annexure A

Pseudocode Writing Task (Algorithm Design)

Code of Conduct:

I Maniyam Anushka / 192525411 (Name / Reg No) certify that this submission is my original work

and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Maniyam Anushka

Name: Maniyam Anushka

Register No: 192525411

ANSWERS

Question 1: Throughput and Packet Loss Calculation

Given: Total data received = 8000 bits, Time = 4 seconds, Packets sent = 200, Packets received = 180.

Pseudocode:

BEGIN

INPUT total_bits_received, total_time, packets_sent, packets_received

// Step 1: Validate inputs to avoid division by zero

IF total_time = 0 OR packets_sent = 0 THEN

DISPLAY "Error: Time and total packets sent must not be zero"

EXIT

ENDIF

// Step 2: Calculate throughput

throughput = total_bits_received / total_time

// Step 3: Calculate packet loss percentage

packets_lost = packets_sent - packets_received

packet_loss_percent = (packets_lost / packets_sent) * 100

// Step 4: Display results

DISPLAY "Throughput = ", throughput, " bps"

DISPLAY "Packet Loss = ", packet_loss_percent, " %"

// Step 5: Classify network performance

IF throughput > 1500 AND packet_loss_percent < 10 THEN

network_status = "Efficient"

ELSE

network_status = "Inefficient"

ENDIF

// Step 6: Display final summary

DISPLAY "----- Network Performance Summary -----"

DISPLAY "Throughput: ", throughput, " bps"

DISPLAY "Packet Loss: ", packet_loss_percent, " %"

DISPLAY "Network Status: ", network_status

END

Dry Run with Given Values:

throughput = 8000 / 4 = 2000 bps

packets_lost = 200 - 180 = 20

packet_loss_percent = (20 / 200) * 100 = 10 %

Since throughput (2000 bps) > 1500 bps but packet loss (10%) is not less than 10%, the network is classified as Inefficient.

Question 2: Bandwidth Monitoring and Overload Detection

Pseudocode:

BEGIN

INITIALIZE link_list = [Link1, Link2, ..., LinkN]

INITIALIZE bandwidth_usage[N]

SET threshold = 80

SET monitoring_active = TRUE

WHILE monitoring_active = TRUE DO

FOR each link i in link_list DO

bandwidth_usage[i] = GET_CURRENT_UTILIZATION(link_list[i])

DISPLAY "Link ", i, " Utilization: ", bandwidth_usage[i], " %"

IF bandwidth_usage[i] > threshold THEN

DISPLAY "Warning: Link ", i, " is overloaded"

IF backup_link_exists(link_list[i]) = TRUE THEN

SWITCH_TRAFFIC(link_list[i], backup_link_of(link_list[i]))

DISPLAY "Traffic switched to backup link for Link ", i

ELSE

DISPLAY "No backup link available for Link ", i

ENDIF

ENDIF

ENDFOR

WAIT(monitoring_interval)

ENDWHILE

END

Explanation:

This algorithm supports efficient bandwidth management by continuously collecting the utilization of every link in a loop and storing the readings in an array, so the administrator always has up-to-date values instead of a one-time snapshot. The threshold check (utilization > 80%) automatically flags a link before it becomes completely saturated, and the algorithm reacts immediately by redirecting traffic to a backup link. This prevents congestion from building up, avoids sudden link failure, and keeps the network running smoothly without requiring the administrator to manually watch every link at all times.

Question 3: Best Path Selection Using Link Quality Score

Pseudocode:

BEGIN

INITIALIZE branches[] = [B1, B2, B3, B4, B5, B6]

INITIALIZE bandwidth[][], delay[][], utilization[][]

SET weight_bw, weight_delay, weight_util   // importance given to each metric

SET monitoring_active = TRUE

WHILE monitoring_active = TRUE DO

FOR each branch i in branches DO

best_score = -INFINITY

best_link = NULL

FOR each link j connecting HQ to branch i DO

bandwidth[i][j] = GET_BANDWIDTH(link[i][j])

delay[i][j] = GET_DELAY(link[i][j])

utilization[i][j] = GET_UTILIZATION(link[i][j])

IF utilization[i][j] > 80 OR STATUS(link[i][j]) = "DOWN" THEN

DISPLAY "Alert: Link ", j, " to Branch ", i, " is overloaded or has failed"

SEND_ALERT_TO_ADMIN(branch i, link[i][j])

CONTINUE   // skip this link, do not consider it for selection

ENDIF

// Step: Calculate link quality score (higher bandwidth and lower delay/utilization is better)

score = (weight_bw * bandwidth[i][j]) - (weight_delay * delay[i][j]) - (weight_util * utilization[i][j])

IF score > best_score THEN

best_score = score

best_link = link[i][j]

ENDIF

ENDFOR

IF best_link = NULL THEN

DISPLAY "No healthy path available for Branch ", i

ELSE

DISPLAY "Best path for Branch ", i, " is ", best_link, " with score ", best_score

ENDIF

ENDFOR

WAIT(monitoring_interval)

ENDWHILE

END

Explanation:

This algorithm improves network reliability by continuously monitoring bandwidth, delay, and utilization for every link instead of checking only once, so it adapts automatically as conditions change throughout the day. It improves routing efficiency because the path is chosen using a combined quality score built from multiple metrics, rather than just the shortest or first available path, which means traffic always favors the link that currently performs best overall. It improves fault tolerance because any link that is overloaded or down is detected immediately, skipped during selection, reported to the administrator through an alert, and automatically replaced with the next best healthy alternate path — keeping communication between headquarters and each branch active with minimal manual intervention.